# Lab 05 – Worksheet

| Name: Muhammad Zubair Asif | ID: ma11925 | Section: T1 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Figure 1: Finite State Machine (FSM) State Transition Diagram

FSM:

Assumptions and Logic:
1. FSM Architecture: Designed as a 2-state synchronous Finite State Machine with states:
   - S_WAIT (1'b0): System monitors the 16 switch inputs. Counter and LEDs remain 0 while switches == 0.
   - S_COUNT (1'b1): Decrement counter is actively decrementing from the captured switch value down to zero cycle-by-cycle.
2. Input Latching: When switches become non-zero in S_WAIT, the switch value is immediately captured into the 16-bit register 'counter' and displayed on 'leds', transitioning to S_COUNT on the next clock edge.
3. Switch Immunity: While in S_COUNT, switch inputs are ignored. The counter and LEDs decrement autonomously until counter reaches zero.
4. Automatic Return: When counter reaches 16'd0, next_state evaluates to S_WAIT, automatically returning the FSM to the input-waiting state.
5. Asynchronous Reset: An active-high reset signal (rst) forces the state register to S_WAIT and clears counter and leds to 0 from any state.

Comments

## Task 2

1. Core FSM Decrement Counter Module (fsm_counter.v):

`timescale 1ns / 1ps
//////////////////////////////////////////////////////////////////////////////////
// Company: 
// Engineer: Muhammad Zubair Asif
// 
// Create Date: 09/23/2026 07:54:07 PM
// Design Name: 
// Module Name: fsm_counter
// Project Name: CompArch F26 lab-05
// Target Devices: BASYS-3
// Tool Versions: 
// Description: 
// 
// Dependencies: 
// 
// Revision:
// Revision 0.01 - File Created
// Additional Comments:
// 
//////////////////////////////////////////////////////////////////////////////////


module fsm_counter(
input wire clk,
input wire rst,
input wire [15:0]switches,
output reg [15:0]leds
);
    localparam S_WAIT = 1'b0;
    localparam S_COUNT = 1'b1;
    
    reg current_state, next_state;
    reg [15:0] counter;
    
    always @(*) begin
    
        case (current_state)
            S_WAIT: next_state = (switches != 16'd0)? S_COUNT: S_WAIT;
        
            S_COUNT: next_state = (counter == 16'd0)? S_WAIT: S_COUNT;
            
            default: next_state = S_WAIT;
        
    endcase   
                
    end
    
    always @(posedge clk or posedge rst) begin
        current_state <= rst? S_WAIT: next_state;
    end
    
    always @(posedge clk or posedge rst) begin
    
        if (rst) begin
            counter <= 16'd0;
            leds <= 16'd0;
        end else begin
            case (current_state)
            
            S_WAIT: begin
                if (switches != 16'd0) begin
                    counter <= switches;
                    leds <= switches;
                end else begin
                    counter <= 16'd0;
                    leds <= 16'd0;
                end
            end
            
            S_COUNT: begin
                if (counter > 16'd0) begin
                    counter <= counter - 1'b1;
                    leds <= counter - 1'b1;
                end else begin
                    counter <= 16'd0;
                    leds <= 16'd0;
                end
            end
                
            default: begin
                counter <= 16'd0;
                leds <= 16'd0;
            end
        endcase
    end
end 
    
endmodule

2. Integrated Top-Level System Module (skeleton_top_fsm.v):

`timescale 1ns / 1ps
//////////////////////////////////////////////////////////////////////////////////
// Company: 
// Engineer: Muhammad Zubair Asif
// 
// Create Date: 09/24/2026 01:00:00 AM
// Design Name: 
// Module Name: skeleton_top_fsm
// Project Name: CompArch F26 lab-05
// Target Devices: BASYS-3
// Tool Versions: 
// Description: 
// 
// Dependencies: 
// 
// Revision:
// Revision 0.01 - File Created 
// Additional Comments:
// 
//////////////////////////////////////////////////////////////////////////////////

module skeleton_top_fsm (
    input wire clk,
    input wire pbin,
    input wire [15:0] physical_sw,
    output wire [15:0] physical_leds
);

    // DEBOUNCER (Cleans up the physical reset button signal)
    wire rst_clean;
    wire [31:0] switch_data; // hold the value read from the switches
    reg [31:0] led_write_data = 32'd0; // counter value here
    wire slow_clk;
  
    debouncer rst_db (
        .clk(clk),
        .pbin(pbin), 
        .pbout(rst_clean) // generated a clean signal
    ); 

    leds switch_reader (
        .clk(clk), 
        .rst(rst_clean),
        .btns(16'd0),        // Not used for this FSM
        .writeData(32'd0),   // We don't write to switches
        .writeEnable(1'b0),  // Disabled
        .readEnable(1'b1),   // Always ON so we can monitor switches
        .memAddress(30'd0),       
        .switches(physical_sw), // Plug in the physical switches
        .readData(switch_data)  // output data 
    );
    
    switches led_writer (
        .clk(clk), 
        .rst(rst_clean),
        .writeData(led_write_data),
        .writeEnable(1'b1),  // Always ON so LEDs update instantly
        .readEnable(1'b0), 
        .memAddress(30'd0),
        .readData(),         // Ignored
        .leds(physical_leds)      
    );
    
    clock_divider ticker (
        .clk_in(clk),       // Feed it the 100MHz fast clock
        .rst(rst_clean),    // Feed it the clean reset signal
        .clk_out(slow_clk)  // It spits out the slow clock
    );

    // YOUR FSM AND COUNTER LOGIC
    wire [15:0] fsm_leds;

    fsm_counter counter_inst (
        .clk(slow_clk),
        .rst(rst_clean),
        .switches(switch_data[15:0]),
        .leds(fsm_leds)
    );

    always @(*) begin
        led_write_data = {16'd0, fsm_leds};
    end

endmodule

3. Core FSM Testbench (fsm_counter_tb.v):

`timescale 1ns / 1ps
//////////////////////////////////////////////////////////////////////////////////
// Company: 
// Engineer: Muhammad Zubair Asif
// 
// Create Date: 09/23/2026 07:54:07 PM
// Design Name: 
// Module Name: fsm_counter_tb
// Project Name: CompArch F26 lab-05
// Target Devices: BASYS-3
// Tool Versions: 
// Description: 
// 
// Dependencies: 
// 
// Revision:
// Revision 0.01 - File Created
// Additional Comments:
// 
//////////////////////////////////////////////////////////////////////////////////


module fsm_counter_tb;

    // Inputs to DUT
    reg clk;
    reg rst;
    reg [15:0] switches;

    // Outputs from DUT
    wire [15:0] leds;

    // Instantiate Device Under Test (DUT)
    fsm_counter uut (
        .clk(clk),
        .rst(rst),
        .switches(switches),
        .leds(leds)
    );

    // Clock generation: 100 MHz (Period = 10ns)
    initial begin
        clk = 0;
        forever #5 clk = ~clk;
    end

    // Monitor countdown progress
    always @(posedge clk) begin
        if (uut.current_state == 1'b1) begin
            $display("    [State S_COUNT] LEDs = %0d | Counter = %0d", leds, uut.counter);
        end
    end

    // Test stimulus sequence
    initial begin
        $timeformat(-9, 0, " ns", 6);

        // Initialize inputs
        rst = 0;
        switches = 16'd0;

        $display("Starting FSM Counter Testbench...");

        // ------------------------------------------------------------
        // Test 1: Power-on Reset
        // ------------------------------------------------------------
        $display("[Time %0t] Test 1: Asserting reset...", $time);
        #10;
        rst = 1;
        #20;
        rst = 0;
        #20;

        // ------------------------------------------------------------
        // Test 2: Idle State Check (switches = 0)
        // ------------------------------------------------------------
        $display("[Time %0t] Test 2: Checking idle state with switches = 0", $time);
        switches = 16'd0;
        #40;

        // ------------------------------------------------------------
        // Test 3: Normal Countdown (Load 5)
        // ------------------------------------------------------------
        $display("[Time %0t] Test 3: Setting switches = 5, triggering countdown", $time);
        @(negedge clk);
        switches = 16'd5;

        // Once captured into S_COUNT, drop switches back to 0
        @(posedge clk);
        #1;
        switches = 16'd0;

        // Wait enough cycles for counter to count: 5 -> 4 -> 3 -> 2 -> 1 -> 0 -> S_WAIT
        #80;

        // ------------------------------------------------------------
        // Test 4: Switch Immunity (Load 6, change switches during countdown)
        // ------------------------------------------------------------
        $display("[Time %0t] Test 4: Testing switch immunity during countdown", $time);
        @(negedge clk);
        switches = 16'd6;
        @(posedge clk);
        #1;
        // Attempt to tamper with switches while counting down
        switches = 16'd15;
        #30;
        switches = 16'd0;
        #50;

        // ------------------------------------------------------------
        // Test 5: Reset Mid-Countdown (Load 10, assert reset halfway)
        // ------------------------------------------------------------
        $display("[Time %0t] Test 5: Testing reset mid-countdown", $time);
        @(negedge clk);
        switches = 16'd10;
        @(posedge clk);
        #1;
        switches = 16'd0;

        // Let it count down a few ticks (10 -> 9 -> 8 -> 7...)
        #30;
        $display("[Time %0t] Asserting reset mid-countdown!", $time);
        rst = 1;
        #15;
        rst = 0;
        #30;

        $display("[Time %0t] All tests completed successfully!", $time);
        $finish;
    end

endmodule

4. Integrated System Testbench (skeleton_top_fsm_tb.v):

Modules + Testbench:

Vivado XSim Simulation Log Output — Core FSM (fsm_counter_tb):

Starting FSM Counter Testbench...
[Time 0 ns] Test 1: Asserting reset...
[Time 50 ns] Test 2: Checking idle state with switches = 0
[Time 90 ns] Test 3: Setting switches = 5, triggering countdown
    [State S_COUNT] LEDs = 5 | Counter = 5
    [State S_COUNT] LEDs = 4 | Counter = 4
    [State S_COUNT] LEDs = 3 | Counter = 3
    [State S_COUNT] LEDs = 2 | Counter = 2
    [State S_COUNT] LEDs = 1 | Counter = 1
    [State S_COUNT] LEDs = 0 | Counter = 0
[Time 176 ns] Test 4: Testing switch immunity during countdown
    [State S_COUNT] LEDs = 6 | Counter = 6
    [State S_COUNT] LEDs = 5 | Counter = 5
    [State S_COUNT] LEDs = 4 | Counter = 4
    [State S_COUNT] LEDs = 3 | Counter = 3
    [State S_COUNT] LEDs = 2 | Counter = 2
    [State S_COUNT] LEDs = 1 | Counter = 1
    [State S_COUNT] LEDs = 0 | Counter = 0
[Time 266 ns] Test 5: Testing reset mid-countdown
    [State S_COUNT] LEDs = 10 | Counter = 10
    [State S_COUNT] LEDs = 9 | Counter = 9
    [State S_COUNT] LEDs = 8 | Counter = 8
[Time 306 ns] Asserting reset mid-countdown!
[Time 351 ns] All tests completed successfully!
$finish called at time : 351 ns

Vivado XSim Simulation Log Output — Top-Level System (skeleton_top_fsm_tb):

Starting Top-Level FSM System Testbench...
[Time 0 ns] Test 1: Asserting reset (pbin)...
[Time 100 ns] Test 2: Checking idle state with physical_sw = 0
[Time 200 ns] Test 3: Setting physical_sw = 5, triggering countdown
    [SlowClk Tick] state=1 | Counter=5 | physical_leds=5
    [SlowClk Tick] state=1 | Counter=4 | physical_leds=4
    [SlowClk Tick] state=1 | Counter=3 | physical_leds=3
    [SlowClk Tick] state=1 | Counter=2 | physical_leds=2
    [SlowClk Tick] state=1 | Counter=1 | physical_leds=1
    [SlowClk Tick] state=1 | Counter=0 | physical_leds=0
[Time 755 ns] Test 4: Testing switch immunity (load 6, tamper with 15)
    [SlowClk Tick] state=1 | Counter=6 | physical_leds=6
    [SlowClk Tick] state=1 | Counter=5 | physical_leds=5
    [SlowClk Tick] state=1 | Counter=4 | physical_leds=4
    [SlowClk Tick] state=1 | Counter=3 | physical_leds=3
    [SlowClk Tick] state=1 | Counter=2 | physical_leds=2
    [SlowClk Tick] state=1 | Counter=1 | physical_leds=1
    [SlowClk Tick] state=1 | Counter=0 | physical_leds=0
[Time 1295 ns] Test 5: Testing reset mid-countdown
    [SlowClk Tick] state=1 | Counter=10 | physical_leds=10
    [SlowClk Tick] state=1 | Counter=9 | physical_leds=9
    [SlowClk Tick] state=1 | Counter=8 | physical_leds=8
[Time 1535 ns] Asserting reset (pbin) mid-countdown!
[Time 1715 ns] All top-level tests completed successfully!
$finish called at time : 1715 ns

Results:

## Task 3

basys3_fsm_counter.xdc (Pin & 100 MHz Clock Constraints):

## ==============================================================================
## Basys-3 FPGA Pin Constraints for Lab 05 (skeleton_top_fsm)
## Target Board: Digilent Basys-3 (xc7a35tcpg236-1)
## Target Module: skeleton_top_fsm
## ==============================================================================

## -----------------------------------------------------------------------------
## 1. Clock Signal (100 MHz Oscillator)
## -----------------------------------------------------------------------------
set_property PACKAGE_PIN W5 [get_ports clk]							
set_property IOSTANDARD LVCMOS33 [get_ports clk]
create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk]

## -----------------------------------------------------------------------------
## 2. Reset Push Button (Center Button - btnC)
## -----------------------------------------------------------------------------
set_property PACKAGE_PIN U18 [get_ports pbin]						
set_property IOSTANDARD LVCMOS33 [get_ports pbin]

## -----------------------------------------------------------------------------
## 3. Physical Switches (physical_sw[15:0])
## -----------------------------------------------------------------------------
set_property PACKAGE_PIN V17 [get_ports {physical_sw[0]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[0]}]
set_property PACKAGE_PIN V16 [get_ports {physical_sw[1]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[1]}]
set_property PACKAGE_PIN W16 [get_ports {physical_sw[2]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[2]}]
set_property PACKAGE_PIN W17 [get_ports {physical_sw[3]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[3]}]
set_property PACKAGE_PIN W15 [get_ports {physical_sw[4]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[4]}]
set_property PACKAGE_PIN V15 [get_ports {physical_sw[5]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[5]}]
set_property PACKAGE_PIN W14 [get_ports {physical_sw[6]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[6]}]
set_property PACKAGE_PIN W13 [get_ports {physical_sw[7]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[7]}]
set_property PACKAGE_PIN V2  [get_ports {physical_sw[8]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[8]}]
set_property PACKAGE_PIN T3  [get_ports {physical_sw[9]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[9]}]
set_property PACKAGE_PIN T2  [get_ports {physical_sw[10]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[10]}]
set_property PACKAGE_PIN R3  [get_ports {physical_sw[11]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[11]}]
set_property PACKAGE_PIN W2  [get_ports {physical_sw[12]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[12]}]
set_property PACKAGE_PIN U1  [get_ports {physical_sw[13]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[13]}]
set_property PACKAGE_PIN T1  [get_ports {physical_sw[14]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[14]}]
set_property PACKAGE_PIN R2  [get_ports {physical_sw[15]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[15]}]

## -----------------------------------------------------------------------------
## 4. Physical LEDs (physical_leds[15:0])
## -----------------------------------------------------------------------------
set_property PACKAGE_PIN U16 [get_ports {physical_leds[0]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[0]}]
set_property PACKAGE_PIN E19 [get_ports {physical_leds[1]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[1]}]
set_property PACKAGE_PIN U19 [get_ports {physical_leds[2]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[2]}]
set_property PACKAGE_PIN V19 [get_ports {physical_leds[3]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[3]}]
set_property PACKAGE_PIN W18 [get_ports {physical_leds[4]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[4]}]
set_property PACKAGE_PIN U15 [get_ports {physical_leds[5]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[5]}]
set_property PACKAGE_PIN U14 [get_ports {physical_leds[6]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[6]}]
set_property PACKAGE_PIN V14 [get_ports {physical_leds[7]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[7]}]
set_property PACKAGE_PIN V13 [get_ports {physical_leds[8]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[8]}]
set_property PACKAGE_PIN V3  [get_ports {physical_leds[9]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[9]}]
set_property PACKAGE_PIN W3  [get_ports {physical_leds[10]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[10]}]
set_property PACKAGE_PIN U3  [get_ports {physical_leds[11]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[11]}]
set_property PACKAGE_PIN P3  [get_ports {physical_leds[12]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[12]}]
set_property PACKAGE_PIN N3  [get_ports {physical_leds[13]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[13]}]
set_property PACKAGE_PIN P1  [get_ports {physical_leds[14]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[14]}]
set_property PACKAGE_PIN L1  [get_ports {physical_leds[15]}]					
set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[15]}]

## -----------------------------------------------------------------------------
## 5. Configuration & Voltage Settings for Basys-3
## -----------------------------------------------------------------------------
set_property CFGBVS VCCO [current_design]
set_property CONFIG_VOLTAGE 3.3 [current_design]

## XDC file

Vivado Static Timing Analysis Summary Report (skeleton_top_fsm):

------------------------------------------------------------------------------------------------
| Design Timing Summary (Target: Basys-3 xc7a35tcpg236-1 @ 100 MHz, Design: skeleton_top_fsm)
------------------------------------------------------------------------------------------------
    WNS(ns)      TNS(ns)  TNS Failing Endpoints  WHS(ns)      THS(ns)  THS Failing Endpoints     WPWS(ns)
      5.983        0.000                      0    0.117        0.000                      0        4.500

All user specified timing constraints are met.
Clock: sys_clk | Period: 10.000 ns | Frequency: 100.000 MHz | Target Device: xc7a35tcpg236-1

Timing Summary

Timing Slack Analysis and Interpretation:
- Target Clock Frequency: 100.000 MHz (Clock Period = 10.000 ns)
- Worst Negative Slack (WNS): +5.983 ns
- Worst Hold Slack (WHS): +0.117 ns
- Total Negative Slack (TNS): 0.000 ns (0 failing endpoints out of 27)
- Total Hold Slack (THS): 0.000 ns (0 failing endpoints out of 27)
- Maximum Data Path Delay: Required Time (10.000 ns) - Slack (5.983 ns) = 4.017 ns

Conclusion: PASS
Because the Worst Negative Slack (WNS) is strictly positive (+5.983 ns), all setup and hold timing constraints are met with a substantial safety margin of ~59.8% of the clock period. The entire top-level integrated design runs reliably at 100 MHz on the Basys-3 Artix-7 FPGA.

Interpretation:

## Task 4

RTL Schematic Architecture and Module Hierarchy:
1. Debouncer (rst_db): Filters mechanical bounce on center push-button pbin (pin U18) to provide clean reset rst_clean.
2. Switch Reader (switch_reader / leds.v): Memory-mapped interface sampling 16 physical switches into 32-bit bus switch_data.
3. Clock Divider (ticker): 26-bit synchronous counter dividing 100 MHz system clock down to slow_clk (~1 Hz for board operation).
4. FSM Counter (counter_inst): Core 2-state FSM (S_WAIT, S_COUNT) with 16-bit decrement datapath and state registers.
5. LED Writer (led_writer / switches.v): Latches lower 16 bits of led_write_data to drive physical LEDs (U16..L1).

## RTL Schematic:

Hardware Resource Utilization (Artix-7 xc7a35tcpg236-1):
+-------------------------+------+-------+-----------+-------+
|        Site Type        | Used | Fixed | Available | Util% |
+-------------------------+------+-------+-----------+-------+
| Slice LUTs              |   75 |     0 |     20800 |  0.36 |
| Slice Registers         |   60 |     0 |     41600 |  0.14 |
|   - Register as Flip-Flop (FDCE/FDRE): 60                  |
|   - Register as Latch (LDCE)         :  0 (0.00%)          |
| Bonded IOB (Pins)       |   34 |     0 |       106 | 32.08 |
| Clock Buffers (BUFG)    |    1 |     0 |        32 |  3.13 |
| Carry Logic (CARRY4)    |   11 |     0 |      5200 |  0.21 |
+-------------------------+------+-------+-----------+-------+
Total Failing Endpoints: 0 | Total Timing Errors: 0 | Latches Inferred: 0

Hardware Verification Summary:
- Clean synthesis completed with 0 errors, 0 critical warnings, and 0 warnings.
- Exactly 60 flip-flops inferred (17 in FSM/counter + 26 clock divider + 16 LED writer + 1 state register); 0 latches inferred.
- Pin assignments mapped all 16 switches (V17..R2), 16 LEDs (U16..L1), clock (W5), and reset button (U18).

Results

# Lab 05 – Designing FSM Using FPGA Switches and LEDs

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Prelab |  | /15 |  | /10 & /10 |
| Task 1 | - | - | /10 |  |
| Task 2 | /15 | /10 |  |  |
| Task 3 |  | /15 |  |  |
| Task 4 |  | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |