# Lab 08 – Worksheet

| Name: Muhammad Zubair Asif | ID: ma11925 | Section: T1 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Block Diagram
a) Data Memory Module

b) Address Decoder

c) Top Module

a) Data Memory Module (DataMemory.v): 512 x 32 RAM array; synchronous write on posedge clk gated by MemWrite; asynchronous combinational read gated by MemRead (single-cycle timing). b) Address Decoder (AddressDecoder.v): Combinational decoder evaluating address[9:8] to generate mutually exclusive device enables (DataMemWrite, DataMemRead, LEDWrite, SwitchReadEnable). c) Top Module (addressDecoderTop.v): Integrates CPU memory bus with Data Memory, MMIO LED interface, MMIO Switch interface, and the Read Data Multiplexer.

Comments

1. Memory Mapping & Addressing Logic: The processor presents a 32-bit address, but only the 10 least significant bits address[9:0] are decoded locally to form a 1024-word memory space. Bits address[9:8] select the device: 2'b00 selects Data Memory (locations 0-511, read/write); 2'b01 selects LEDs (locations 512-767, write-only); 2'b10 selects Switches (locations 768-1023, read-only); 2'b11 is unused. 2. Timing Conventions: Write operations are synchronous to posedge clk to guarantee stable data capture. Memory read is asynchronous (combinational) to satisfy single-cycle RISC-V timing, allowing the LW instruction to retrieve data in the same clock cycle. 3. Mutual Exclusion & Isolation: Gated enable signals guarantee mutual exclusion, preventing bus contention. Disallowed transactions (writes to switch addresses or reads from LED addresses) are safely ignored and return zero.

## Task 2

Modules for DataMemory, AddressDecoder, AddressDecoderTop, Switches and Leds.

// ==================== 1. DataMemory.v ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 10/15/2026 03:00:00 PM // Design Name: // Module Name: DataMemory // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module DataMemory #( parameter DEPTH = 512, parameter DATA_WIDTH = 32, parameter ADDR_WIDTH = 9 )( input wire clk, input wire rst, input wire MemWrite, input wire MemRead, input wire [ADDR_WIDTH-1:0] address, input wire [DATA_WIDTH-1:0] write_data, output wire [DATA_WIDTH-1:0] read_data ); // 512 locations of 32-bit memory array reg [DATA_WIDTH-1:0] memory [0:DEPTH-1]; integer i; // Initialize all memory locations to 0 initial begin for (i = 0; i < DEPTH; i = i + 1) begin memory[i] = {DATA_WIDTH{1'b0}}; end end // Synchronous write and synchronous reset always @(posedge clk) begin if (rst) begin for (i = 0; i < DEPTH; i = i + 1) begin memory[i] <= {DATA_WIDTH{1'b0}}; end end else if (MemWrite) begin memory[address] <= write_data; end end // Asynchronous read (single-cycle processor convention) // If MemRead is high, return data at address; otherwise output 0 assign read_data = (MemRead) ? memory[address] : {DATA_WIDTH{1'b0}}; endmodule // ==================== 2. AddressDecoder.v ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 10/15/2026 03:05:00 PM // Design Name: // Module Name: AddressDecoder // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module AddressDecoder ( input wire [31:0] address, input wire readEnable, input wire writeEnable, output wire DataMemWrite, output wire DataMemRead, output wire LEDWrite, output wire SwitchReadEnable, output wire DataMemSelect, output wire LEDSelect, output wire SwitchSelect ); // Device selection based on address[9:8] (Lab 8 Manual Table) assign DataMemSelect = (address[9:8] == 2'b00); assign LEDSelect = (address[9:8] == 2'b01); assign SwitchSelect = (address[9:8] == 2'b10); // Gated enables ensuring mutual exclusion and preventing bus contention // 1. Data Memory: supports both read and write assign DataMemWrite = writeEnable && DataMemSelect; assign DataMemRead = readEnable && DataMemSelect; // 2. LEDs: write-only interface assign LEDWrite = writeEnable && LEDSelect; // 3. Switches: read-only interface assign SwitchReadEnable = readEnable && SwitchSelect; endmodule // ==================== 3. addressDecoderTop.v ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 10/15/2026 03:10:00 PM // Design Name: // Module Name: addressDecoderTop // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module addressDecoderTop ( input wire clk, input wire rst, input wire [31:0] address, input wire readEnable, input wire writeEnable, input wire [31:0] writeData, input wire [15:0] switches, output wire [31:0] readData, output wire [15:0] leds ); // 1. Decoder Enable & Select Signals wire DataMemWrite; wire DataMemRead; wire LEDWrite; wire SwitchReadEnable; wire DataMemSelect; wire LEDSelect; wire SwitchSelect; AddressDecoder decoder_inst ( .address(address), .readEnable(readEnable), .writeEnable(writeEnable), .DataMemWrite(DataMemWrite), .DataMemRead(DataMemRead), .LEDWrite(LEDWrite), .SwitchReadEnable(SwitchReadEnable), .DataMemSelect(DataMemSelect), .LEDSelect(LEDSelect), .SwitchSelect(SwitchSelect) ); // 2. Data Memory Instantiation (512 locations x 32 bits) wire [31:0] data_mem_out; DataMemory #( .DEPTH(512), .DATA_WIDTH(32), .ADDR_WIDTH(9) ) data_mem_inst ( .clk(clk), .rst(rst), .MemWrite(DataMemWrite), .MemRead(DataMemRead), .address(address[8:0]), .write_data(writeData), .read_data(data_mem_out) ); // 3. Switch Interface Instantiation (Lab 5 switch module) wire [31:0] switch_data_out; switches switch_interface ( .physical_sw(switches), .switch_data(switch_data_out) ); // 4. LED Interface Instantiation (Lab 5 leds module) // To allow leds.v to retain its state when LEDWrite is not asserted, // feed back the current LED output unless a write occurs. wire [15:0] leds_internal; wire [31:0] led_write_bus = (LEDWrite) ? writeData : {16'd0, leds_internal}; leds led_interface ( .clk(clk), .rst(rst), .writeData(led_write_bus), .physical_leds(leds_internal) ); assign leds = leds_internal; // 5. Read Data Multiplexer // Routes DataMemory or Switches to CPU readData, defaulting to 0 on inactive/invalid reads. assign readData = (DataMemSelect && DataMemRead) ? data_mem_out : (SwitchSelect && SwitchReadEnable) ? switch_data_out : 32'd0; endmodule // ==================== 4. switches.v ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: Muddassir Ali // // Module Name: switches // Project Name: Counter // Target Devices: Baasys 3 // ////////////////////////////////////////////////////////////////////////////////// module switches( input [15:0] physical_sw, output [31:0] switch_data ); assign switch_data = {16'd0, physical_sw}; endmodule // ==================== 5. leds.v ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: Muddassir Ali // // Module Name: leds // Project Name: Counter // Target Devices: Baasys 3 // ////////////////////////////////////////////////////////////////////////////////// module leds( input clk, input rst, input [31:0] writeData, output reg [15:0] physical_leds ); always @(posedge clk) begin if (rst) physical_leds <= 16'd0; else physical_leds <= writeData[15:0]; end endmodule

Testbench:

// ==================== MemorySystem_tb.v (Self-Checking Testbench) ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 10/15/2026 03:15:00 PM // Design Name: // Module Name: MemorySystem_tb // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module MemorySystem_tb; reg clk; reg rst; reg [31:0] address; reg readEnable; reg writeEnable; reg [31:0] writeData; reg [15:0] switches; wire [31:0] readData; wire [15:0] leds; integer errors = 0; // Instantiate Device Under Test (DUT) addressDecoderTop dut ( .clk(clk), .rst(rst), .address(address), .readEnable(readEnable), .writeEnable(writeEnable), .writeData(writeData), .switches(switches), .readData(readData), .leds(leds) ); // Clock generation: 100MHz (10ns period) always #5 clk = ~clk; initial begin $dumpfile("waveform.vcd"); $dumpvars(0, MemorySystem_tb); $display("==================================================================="); $display(" Starting Lab 8 Memory System & Address Decoder Tests "); $display("==================================================================="); // Initial setup clk = 0; rst = 1; address = 32'd0; readEnable = 0; writeEnable = 0; writeData = 32'd0; switches = 16'h0000; // Apply reset for 2 clock cycles #20; rst = 0; #10; // ------------------------------------------------------------------------- // TEST 1: Data Memory Write & Read Back (address[9:8] == 2'b00) // ------------------------------------------------------------------------- $display("\n--- TEST 1: Data Memory Write and Read-Back ---"); // Write to address 0 (address[9:8] = 00, local 0) @(posedge clk); address = 32'h0000_0000; writeData = 32'hDEADBEEF; writeEnable = 1; readEnable = 0; @(posedge clk); writeEnable = 0; // Write to address 10 (address[9:8] = 00, local 10) @(posedge clk); address = 32'h0000_000A; writeData = 32'hCAFE1234; writeEnable = 1; @(posedge clk); writeEnable = 0; // Write to address 255 (address[9:8] = 00, local 255) @(posedge clk); address = 32'h0000_00FF; writeData = 32'h55AA33CC; writeEnable = 1; @(posedge clk); writeEnable = 0; // Read back from address 0 @(posedge clk); address = 32'h0000_0000; readEnable = 1; #1; // combinational read settling if (readData === 32'hDEADBEEF) begin $display("[PASS] Read Memory[0]: Expected 32'hDEADBEEF, Got 32'h%h", readData); end else begin $display("[FAIL] Read Memory[0]: Expected 32'hDEADBEEF, Got 32'h%h", readData); errors = errors + 1; end // Read back from address 10 @(posedge clk); address = 32'h0000_000A; readEnable = 1; #1; if (readData === 32'hCAFE1234) begin $display("[PASS] Read Memory[10]: Expected 32'hCAFE1234, Got 32'h%h", readData); end else begin $display("[FAIL] Read Memory[10]: Expected 32'hCAFE1234, Got 32'h%h", readData); errors = errors + 1; end // Read back from address 255 @(posedge clk); address = 32'h0000_00FF; readEnable = 1; #1; if (readData === 32'h55AA33CC) begin $display("[PASS] Read Memory[255]: Expected 32'h55AA33CC, Got 32'h%h", readData); end else begin $display("[FAIL] Read Memory[255]: Expected 32'h55AA33CC, Got 32'h%h", readData); errors = errors + 1; end readEnable = 0; // ------------------------------------------------------------------------- // TEST 2: Memory-Mapped LEDs Write (address[9:8] == 2'b01) // ------------------------------------------------------------------------- $display("\n--- TEST 2: Memory-Mapped LEDs Write ---"); // Write 0xABCD to LEDs (address[9:8] = 2'b01 -> address = 0x100 or 0x110) @(posedge clk); address = 32'h0000_0100; // address[9:8] = 2'b01 writeData = 32'h0000_ABCD; writeEnable = 1; readEnable = 0; @(posedge clk); #1; if (leds === 16'hABCD) begin $display("[PASS] Write LEDs (0xABCD): LEDs correctly show 16'h%h", leds); end else begin $display("[FAIL] Write LEDs (0xABCD): Expected 16'hABCD, Got 16'h%h", leds); errors = errors + 1; end // Overwrite LEDs with 0x1234 at another LED offset (e.g. 0x110) @(posedge clk); address = 32'h0000_0110; // address[9:8] = 2'b01 writeData = 32'hFFFF_1234; writeEnable = 1; @(posedge clk); #1; if (leds === 16'h1234) begin $display("[PASS] Overwrite LEDs (0x1234): LEDs correctly show 16'h%h", leds); end else begin $display("[FAIL] Overwrite LEDs (0x1234): Expected 16'h1234, Got 16'h%h", leds); errors = errors + 1; end writeEnable = 0; // ------------------------------------------------------------------------- // TEST 3: Memory-Mapped Switches Read (address[9:8] == 2'b10) // ------------------------------------------------------------------------- $display("\n--- TEST 3: Memory-Mapped Switches Read ---"); // Set physical switch pattern 1 switches = 16'hF0F0; @(posedge clk); address = 32'h0000_0200; // address[9:8] = 2'b10 readEnable = 1; writeEnable = 0; #1; if (readData === 32'h0000_F0F0) begin $display("[PASS] Read Switches (0xF0F0): CPU readData = 32'h%h", readData); end else begin $display("[FAIL] Read Switches (0xF0F0): Expected 32'h0000F0F0, Got 32'h%h", readData); errors = errors + 1; end // Change switch pattern 2 switches = 16'hA55A; #1; if (readData === 32'h0000_A55A) begin $display("[PASS] Read Switches dynamic change (0xA55A): CPU readData = 32'h%h", readData); end else begin $display("[FAIL] Read Switches dynamic change (0xA55A): Expected 32'h0000A55A, Got 32'h%h", readData); errors = errors + 1; end readEnable = 0; // ------------------------------------------------------------------------- // TEST 4: Address Decoder Isolation & Contention Verification // ------------------------------------------------------------------------- $display("\n--- TEST 4: Decoder Isolation & Mutual Exclusion ---"); // 4A: Write to Data Memory must NOT alter LEDs @(posedge clk); address = 32'h0000_0005; // Memory address writeData = 32'h9999_9999; writeEnable = 1; @(posedge clk); #1; if (leds === 16'h1234) begin $display("[PASS] Memory write does NOT corrupt LEDs: LEDs remain 16'h%h", leds); end else begin $display("[FAIL] Memory write corrupted LEDs! LEDs = 16'h%h", leds); errors = errors + 1; end // 4B: Attempt to write to Switch address (read-only) must be ignored @(posedge clk); address = 32'h0000_0200; // Switch address writeData = 32'hDEAD_BEEF; writeEnable = 1; @(posedge clk); #1; if (leds === 16'h1234) begin $display("[PASS] Write to read-only Switch address is safely ignored"); end else begin $display("[FAIL] Write to Switch address leaked to LEDs! LEDs = 16'h%h", leds); errors = errors + 1; end // 4C: Attempt to read from LED address (write-only) should return 0 @(posedge clk); address = 32'h0000_0100; // LED address writeEnable = 0; readEnable = 1; #1; if (readData === 32'd0) begin $display("[PASS] Read from write-only LED address returns 32'h00000000"); end else begin $display("[FAIL] Read from LED address returned non-zero: 32'h%h", readData); errors = errors + 1; end // 4D: Access to unused address space (address[9:8] == 2'b11) returns 0 @(posedge clk); address = 32'h0000_0300; // Unused address (2'b11) writeEnable = 1; writeData = 32'hFFFF_FFFF; readEnable = 1; @(posedge clk); #1; if (readData === 32'd0 && leds === 16'h1234) begin $display("[PASS] Access to Unused address space (2'b11) has no side-effects and returns 0"); end else begin $display("[FAIL] Unused address access caused unwanted side-effects! readData=%h, leds=%h", readData, leds); errors = errors + 1; end // 4E: Read without readEnable returns 0 @(posedge clk); address = 32'h0000_0000; // Memory has DEADBEEF readEnable = 0; writeEnable = 0; #1; if (readData === 32'd0) begin $display("[PASS] Read with readEnable=0 returns 32'h00000000"); end else begin $display("[FAIL] Read with readEnable=0 returned data: 32'h%h", readData); errors = errors + 1; end // ------------------------------------------------------------------------- // Final Summary // ------------------------------------------------------------------------- $display("\n==================================================================="); if (errors == 0) begin $display(" *** ALL TESTS PASSED SUCCESSFULLY! (0 Errors) *** "); $display(" Data Memory, Address Decoder, LEDs, and Switches are verified! "); end else begin $display(" *** TEST FAILED WITH %0d ERROR(S) *** ", errors); end $display("===================================================================\n"); #20; $finish; end endmodule

Waveforms:

Test 1 (Data Memory Write & Read-Back): Verified writing 32'hDEADBEEF, 32'hCAFE1234, and 32'h55AA33CC to addresses 0, 10, and 255 (address[9:8]=00). Subsequent reads accurately returned stored data. Test 2 (MMIO LED Writes): Verified writing patterns 0xABCD and 0x1234 to MMIO LED addresses (address[9:8]=01). Physical LEDs updated immediately and held values when writeEnable was deasserted. Test 3 (MMIO Switch Reads): Verified reading live switch inputs 0xF0F0 and 0xA55A (address[9:8]=10). readData correctly reflected switch values with upper 16 bits zero-extended. Test 4 (Decoder Isolation & Contention): Verified that writes to memory do not affect LEDs, writes to switches are safely ignored, reads from write-only LEDs return 0, and unused addresses (2'b11) produce 0 without side effects. Verification Result: All functional tests passed in 266 ns with 0 Errors in Vivado Simulator (XSim).

## Task 3

## FSM

Hardware Demonstration System: The hardware system top_memory_system wraps addressDecoderTop for testing on the Basys 3 FPGA board. It supports two modes selected by SW[1]: - Automated FSM Mode (SW[1]==0): A 1-second clock ticker (or manual button pbin) cycles through an 8-state sequence: (0) Write 0xAAAA to Mem[0], (1) Write 0x5555 to Mem[1], (2) Write 0x0F0F to MMIO LEDs, (3) Read Mem[0] -> LEDs show 0xAAAA, (4) Read Mem[1] -> LEDs show 0x5555, (5) Read MMIO Switches -> LEDs mirror switches, (6) Store switches into Mem[2], (7) Read back Mem[2]. - Manual Interactive Mode (SW[1]==1): SW[15:14] selects device (00=Mem, 01=LEDs, 10=Switches), SW[13:8] sets local address, SW[7:2] provides write data, and SW[9] selects Read (0) or Write (1, triggered by pbin). - 7-Segment Display: The onboard 7-segment display shows the active state/opcode on digits 0-2, while the decimal point dp lights up when writeEnable is active. // ==================== top_memory_system.v (FPGA Top Module) ==================== `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 10/15/2026 03:20:00 PM // Design Name: // Module Name: top_memory_system // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module top_memory_system #( parameter SIM_DIVIDER_COUNT = 50_000_000 - 1, parameter SIM_REFRESH_BITS = 18 )( input wire clk, input wire pbin, input wire [15:0] physical_sw, output wire [15:0] physical_leds, output wire [6:0] seg, output wire dp, output wire [3:0] an ); // 1. Debounce Push Button & Detect Rising Edge wire btn_clean; reg btn_prev = 0; wire btn_press = (btn_clean && !btn_prev); debouncer btn_db ( .clk(clk), .pbin(pbin), .pbout(btn_clean) ); always @(posedge clk) begin btn_prev <= btn_clean; end // 2. 1Hz Slow Clock Generator for Auto FSM Stepping wire slow_clk; clock_divider #(.MAX_COUNT(SIM_DIVIDER_COUNT)) ticker ( .clk_in(clk), .rst(1'b0), .clk_out(slow_clk) ); reg slow_clk_prev = 0; always @(posedge clk) begin slow_clk_prev <= slow_clk; end wire slow_tick = (slow_clk && !slow_clk_prev); // Auto-stepping enabled when physical_sw[0] == 0 and not paused wire auto_step = (!physical_sw[0]) && slow_tick; wire step_event = auto_step ¦¦ btn_press; // 3. CPU Control Signals & MMIO Wires reg [31:0] cpu_address; reg cpu_read_en; reg cpu_write_en; reg [31:0] cpu_write_data; wire [31:0] cpu_read_data; wire [15:0] mmio_leds; // Instantiate Memory Subsystem (addressDecoderTop) addressDecoderTop mem_subsystem ( .clk(clk), .rst(1'b0), .address(cpu_address), .readEnable(cpu_read_en), .writeEnable(cpu_write_en), .writeData(cpu_write_data), .switches(physical_sw), .readData(cpu_read_data), .leds(mmio_leds) ); // 4. Demonstration FSM States localparam [2:0] S_WRITE_MEM0 = 3'd0, // Write 0xAAAA to Memory[0] S_WRITE_MEM1 = 3'd1, // Write 0x5555 to Memory[1] S_WRITE_LEDS = 3'd2, // Write 0x0F0F to MMIO LEDs (address 0x100) S_READ_MEM0 = 3'd3, // Read Memory[0] (verifies 0xAAAA) S_READ_MEM1 = 3'd4, // Read Memory[1] (verifies 0x5555) S_READ_SW = 3'd5, // Read MMIO Switches (address 0x200) S_WRITE_SWMEM= 3'd6, // Write Switch values into Memory[2] S_READ_SWMEM = 3'd7; // Read back Memory[2] (verifies Switch values) reg [2:0] state = S_WRITE_MEM0; // FSM State Transition always @(posedge clk) begin if (!physical_sw[1]) begin // FSM Mode (physical_sw[1] == 0) if (step_event) begin state <= state + 1; end end else begin // Manual Mode: reset FSM state to 0 for when user returns to FSM state <= S_WRITE_MEM0; end end // FSM / Manual Control Signal Multiplexing always @(*) begin if (physical_sw[1]) begin // Manual Mode: // physical_sw[15:14]: Device select (00=Mem, 01=LEDs, 10=Switches) // physical_sw[13:8] : Local address // physical_sw[7:2] : Write Data [5:0] (expanded) // physical_sw[9] : 1 = Write (on btn_press), 0 = Read cpu_address = {22'd0, physical_sw[15:14], 2'b00, physical_sw[13:8]}; cpu_write_data = {26'd0, physical_sw[7:2]}; cpu_write_en = physical_sw[9] && btn_press; cpu_read_en = !physical_sw[9]; end else begin // Automated FSM Mode: case (state) S_WRITE_MEM0: begin cpu_address = 32'h0000_0000; // Memory[0] (address[9:8] = 00) cpu_write_data = 32'h0000_AAAA; cpu_write_en = 1'b1; cpu_read_en = 1'b0; end S_WRITE_MEM1: begin cpu_address = 32'h0000_0001; // Memory[1] (address[9:8] = 00) cpu_write_data = 32'h0000_5555; cpu_write_en = 1'b1; cpu_read_en = 1'b0; end S_WRITE_LEDS: begin cpu_address = 32'h0000_0100; // LEDs (address[9:8] = 01) cpu_write_data = 32'h0000_0F0F; cpu_write_en = 1'b1; cpu_read_en = 1'b0; end S_READ_MEM0: begin cpu_address = 32'h0000_0000; // Read Memory[0] cpu_write_data = 32'd0; cpu_write_en = 1'b0; cpu_read_en = 1'b1; end S_READ_MEM1: begin cpu_address = 32'h0000_0001; // Read Memory[1] cpu_write_data = 32'd0; cpu_write_en = 1'b0; cpu_read_en = 1'b1; end S_READ_SW: begin cpu_address = 32'h0000_0200; // Read Switches (address[9:8] = 10) cpu_write_data = 32'd0; cpu_write_en = 1'b0; cpu_read_en = 1'b1; end S_WRITE_SWMEM: begin cpu_address = 32'h0000_0002; // Write Switches to Memory[2] cpu_write_data = {16'd0, physical_sw}; cpu_write_en = 1'b1; cpu_read_en = 1'b0; end S_READ_SWMEM: begin cpu_address = 32'h0000_0002; // Read back Memory[2] cpu_write_data = 32'd0; cpu_write_en = 1'b0; cpu_read_en = 1'b1; end endcase end end // 5. Physical LEDs Display Logic // In FSM Mode: // - During Write LEDs state (or when physical_sw[8]==1), display mmio_leds // - During Read states, display the cpu_read_data[15:0] assign physical_leds = (physical_sw[8] ¦¦ (state == S_WRITE_LEDS)) ? mmio_leds : cpu_read_data[15:0]; // 6. Seven-Segment Display Controller // Displays the current FSM state / mode on an[2:0] // dp lights up when a write operation is active seven_seg_controller #(.REFRESH_BITS(SIM_REFRESH_BITS)) seg_display ( .clk(clk), .rst(1'b0), .opcode(state), .zero_flag(cpu_write_en), .seg(seg), .dp(dp), .an(an) ); endmodule

Constraints File (cons_1.xdc):

// ==================== cons_1.xdc (Basys 3 Constraints) ==================== ## This file is a general .xdc for the Basys3 rev B board ## To use it in a project: ## - uncomment the lines corresponding to used pins ## - rename the used ports (in each line, after get_ports) according to the top level signal names in the project ## Clock signal set_property -dict { PACKAGE_PIN W5 IOSTANDARD LVCMOS33 } [get_ports clk] create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk] ## Switches set_property -dict { PACKAGE_PIN V17 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[0]}] set_property -dict { PACKAGE_PIN V16 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[1]}] set_property -dict { PACKAGE_PIN W16 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[2]}] set_property -dict { PACKAGE_PIN W17 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[3]}] set_property -dict { PACKAGE_PIN W15 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[4]}] set_property -dict { PACKAGE_PIN V15 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[5]}] set_property -dict { PACKAGE_PIN W14 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[6]}] set_property -dict { PACKAGE_PIN W13 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[7]}] set_property -dict { PACKAGE_PIN V2 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[8]}] set_property -dict { PACKAGE_PIN T3 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[9]}] set_property -dict { PACKAGE_PIN T2 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[10]}] set_property -dict { PACKAGE_PIN R3 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[11]}] set_property -dict { PACKAGE_PIN W2 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[12]}] set_property -dict { PACKAGE_PIN U1 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[13]}] set_property -dict { PACKAGE_PIN T1 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[14]}] set_property -dict { PACKAGE_PIN R2 IOSTANDARD LVCMOS33 } [get_ports {physical_sw[15]}] ## LEDs set_property -dict { PACKAGE_PIN U16 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[0]}] set_property -dict { PACKAGE_PIN E19 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[1]}] set_property -dict { PACKAGE_PIN U19 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[2]}] set_property -dict { PACKAGE_PIN V19 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[3]}] set_property -dict { PACKAGE_PIN W18 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[4]}] set_property -dict { PACKAGE_PIN U15 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[5]}] set_property -dict { PACKAGE_PIN U14 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[6]}] set_property -dict { PACKAGE_PIN V14 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[7]}] set_property -dict { PACKAGE_PIN V13 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[8]}] set_property -dict { PACKAGE_PIN V3 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[9]}] set_property -dict { PACKAGE_PIN W3 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[10]}] set_property -dict { PACKAGE_PIN U3 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[11]}] set_property -dict { PACKAGE_PIN P3 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[12]}] set_property -dict { PACKAGE_PIN N3 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[13]}] set_property -dict { PACKAGE_PIN P1 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[14]}] set_property -dict { PACKAGE_PIN L1 IOSTANDARD LVCMOS33 } [get_ports {physical_leds[15]}] ##7 Segment Display set_property -dict { PACKAGE_PIN W7 IOSTANDARD LVCMOS33 } [get_ports {seg[0]}] set_property -dict { PACKAGE_PIN W6 IOSTANDARD LVCMOS33 } [get_ports {seg[1]}] set_property -dict { PACKAGE_PIN U8 IOSTANDARD LVCMOS33 } [get_ports {seg[2]}] set_property -dict { PACKAGE_PIN V8 IOSTANDARD LVCMOS33 } [get_ports {seg[3]}] set_property -dict { PACKAGE_PIN U5 IOSTANDARD LVCMOS33 } [get_ports {seg[4]}] set_property -dict { PACKAGE_PIN V5 IOSTANDARD LVCMOS33 } [get_ports {seg[5]}] set_property -dict { PACKAGE_PIN U7 IOSTANDARD LVCMOS33 } [get_ports {seg[6]}] set_property -dict { PACKAGE_PIN V7 IOSTANDARD LVCMOS33 } [get_ports dp] set_property -dict { PACKAGE_PIN U2 IOSTANDARD LVCMOS33 } [get_ports {an[0]}] set_property -dict { PACKAGE_PIN U4 IOSTANDARD LVCMOS33 } [get_ports {an[1]}] set_property -dict { PACKAGE_PIN V4 IOSTANDARD LVCMOS33 } [get_ports {an[2]}] set_property -dict { PACKAGE_PIN W4 IOSTANDARD LVCMOS33 } [get_ports {an[3]}] ##Buttons set_property -dict { PACKAGE_PIN U18 IOSTANDARD LVCMOS33 } [get_ports pbin] #set_property -dict { PACKAGE_PIN T18 IOSTANDARD LVCMOS33 } [get_ports btnU] #set_property -dict { PACKAGE_PIN W19 IOSTANDARD LVCMOS33 } [get_ports btnL] #set_property -dict { PACKAGE_PIN T17 IOSTANDARD LVCMOS33 } [get_ports btnR] #set_property -dict { PACKAGE_PIN U17 IOSTANDARD LVCMOS33 } [get_ports btnD] ##Pmod Header JA #set_property -dict { PACKAGE_PIN J1 IOSTANDARD LVCMOS33 } [get_ports {JA[0]}];#Sch name = JA1 #set_property -dict { PACKAGE_PIN L2 IOSTANDARD LVCMOS33 } [get_ports {JA[1]}];#Sch name = JA2 #set_property -dict { PACKAGE_PIN J2 IOSTANDARD LVCMOS33 } [get_ports {JA[2]}];#Sch name = JA3 #set_property -dict { PACKAGE_PIN G2 IOSTANDARD LVCMOS33 } [get_ports {JA[3]}];#Sch name = JA4 #set_property -dict { PACKAGE_PIN H1 IOSTANDARD LVCMOS33 } [get_ports {JA[4]}];#Sch name = JA7 #set_property -dict { PACKAGE_PIN K2 IOSTANDARD LVCMOS33 } [get_ports {JA[5]}];#Sch name = JA8 #set_property -dict { PACKAGE_PIN H2 IOSTANDARD LVCMOS33 } [get_ports {JA[6]}];#Sch name = JA9 #set_property -dict { PACKAGE_PIN G3 IOSTANDARD LVCMOS33 } [get_ports {JA[7]}];#Sch name = JA10 ##Pmod Header JB #set_property -dict { PACKAGE_PIN A14 IOSTANDARD LVCMOS33 } [get_ports {JB[0]}];#Sch name = JB1 #set_property -dict { PACKAGE_PIN A16 IOSTANDARD LVCMOS33 } [get_ports {JB[1]}];#Sch name = JB2 #set_property -dict { PACKAGE_PIN B15 IOSTANDARD LVCMOS33 } [get_ports {JB[2]}];#Sch name = JB3 #set_property -dict { PACKAGE_PIN B16 IOSTANDARD LVCMOS33 } [get_ports {JB[3]}];#Sch name = JB4 #set_property -dict { PACKAGE_PIN A15 IOSTANDARD LVCMOS33 } [get_ports {JB[4]}];#Sch name = JB7 #set_property -dict { PACKAGE_PIN A17 IOSTANDARD LVCMOS33 } [get_ports {JB[5]}];#Sch name = JB8 #set_property -dict { PACKAGE_PIN C15 IOSTANDARD LVCMOS33 } [get_ports {JB[6]}];#Sch name = JB9 #set_property -dict { PACKAGE_PIN C16 IOSTANDARD LVCMOS33 } [get_ports {JB[7]}];#Sch name = JB10 ##Pmod Header JC #set_property -dict { PACKAGE_PIN K17 IOSTANDARD LVCMOS33 } [get_ports {JC[0]}];#Sch name = JC1 #set_property -dict { PACKAGE_PIN M18 IOSTANDARD LVCMOS33 } [get_ports {JC[1]}];#Sch name = JC2 #set_property -dict { PACKAGE_PIN N17 IOSTANDARD LVCMOS33 } [get_ports {JC[2]}];#Sch name = JC3 #set_property -dict { PACKAGE_PIN P18 IOSTANDARD LVCMOS33 } [get_ports {JC[3]}];#Sch name = JC4 #set_property -dict { PACKAGE_PIN L17 IOSTANDARD LVCMOS33 } [get_ports {JC[4]}];#Sch name = JC7 #set_property -dict { PACKAGE_PIN M19 IOSTANDARD LVCMOS33 } [get_ports {JC[5]}];#Sch name = JC8 #set_property -dict { PACKAGE_PIN P17 IOSTANDARD LVCMOS33 } [get_ports {JC[6]}];#Sch name = JC9 #set_property -dict { PACKAGE_PIN R18 IOSTANDARD LVCMOS33 } [get_ports {JC[7]}];#Sch name = JC10 ##Pmod Header JXADC #set_property -dict { PACKAGE_PIN J3 IOSTANDARD LVCMOS33 } [get_ports {JXADC[0]}];#Sch name = XA1_P #set_property -dict { PACKAGE_PIN L3 IOSTANDARD LVCMOS33 } [get_ports {JXADC[1]}];#Sch name = XA2_P #set_property -dict { PACKAGE_PIN M2 IOSTANDARD LVCMOS33 } [get_ports {JXADC[2]}];#Sch name = XA3_P #set_property -dict { PACKAGE_PIN N2 IOSTANDARD LVCMOS33 } [get_ports {JXADC[3]}];#Sch name = XA4_P #set_property -dict { PACKAGE_PIN K3 IOSTANDARD LVCMOS33 } [get_ports {JXADC[4]}];#Sch name = XA1_N #set_property -dict { PACKAGE_PIN M3 IOSTANDARD LVCMOS33 } [get_ports {JXADC[5]}];#Sch name = XA2_N #set_property -dict { PACKAGE_PIN M1 IOSTANDARD LVCMOS33 } [get_ports {JXADC[6]}];#Sch name = XA3_N #set_property -dict { PACKAGE_PIN N1 IOSTANDARD LVCMOS33 } [get_ports {JXADC[7]}];#Sch name = XA4_N ##VGA Connector #set_property -dict { PACKAGE_PIN G19 IOSTANDARD LVCMOS33 } [get_ports {vgaRed[0]}] #set_property -dict { PACKAGE_PIN H19 IOSTANDARD LVCMOS33 } [get_ports {vgaRed[1]}] #set_property -dict { PACKAGE_PIN J19 IOSTANDARD LVCMOS33 } [get_ports {vgaRed[2]}] #set_property -dict { PACKAGE_PIN N19 IOSTANDARD LVCMOS33 } [get_ports {vgaRed[3]}] #set_property -dict { PACKAGE_PIN N18 IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[0]}] #set_property -dict { PACKAGE_PIN L18 IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[1]}] #set_property -dict { PACKAGE_PIN K18 IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[2]}] #set_property -dict { PACKAGE_PIN J18 IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[3]}] #set_property -dict { PACKAGE_PIN J17 IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[0]}] #set_property -dict { PACKAGE_PIN H17 IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[1]}] #set_property -dict { PACKAGE_PIN G17 IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[2]}] #set_property -dict { PACKAGE_PIN D17 IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[3]}] #set_property -dict { PACKAGE_PIN P19 IOSTANDARD LVCMOS33 } [get_ports Hsync] #set_property -dict { PACKAGE_PIN R19 IOSTANDARD LVCMOS33 } [get_ports Vsync] ##USB-RS232 Interface #set_property -dict { PACKAGE_PIN B18 IOSTANDARD LVCMOS33 } [get_ports RsRx] #set_property -dict { PACKAGE_PIN A18 IOSTANDARD LVCMOS33 } [get_ports RsTx] ##USB HID (PS/2) #set_property -dict { PACKAGE_PIN C17 IOSTANDARD LVCMOS33 PULLUP true } [get_ports PS2Clk] #set_property -dict { PACKAGE_PIN B17 IOSTANDARD LVCMOS33 PULLUP true } [get_ports PS2Data] ##Quad SPI Flash ##Note that CCLK_0 cannot be placed in 7 series devices. You can access it using the ##STARTUPE2 primitive. #set_property -dict { PACKAGE_PIN D18 IOSTANDARD LVCMOS33 } [get_ports {QspiDB[0]}] #set_property -dict { PACKAGE_PIN D19 IOSTANDARD LVCMOS33 } [get_ports {QspiDB[1]}] #set_property -dict { PACKAGE_PIN G18 IOSTANDARD LVCMOS33 } [get_ports {QspiDB[2]}] #set_property -dict { PACKAGE_PIN F18 IOSTANDARD LVCMOS33 } [get_ports {QspiDB[3]}] #set_property -dict { PACKAGE_PIN K19 IOSTANDARD LVCMOS33 } [get_ports QspiCSn] ## Configuration options, can be used for all designs set_property CONFIG_VOLTAGE 3.3 [current_design] set_property CFGBVS VCCO [current_design] ## SPI configuration mode options for QSPI boot, can be used for all designs set_property BITSTREAM.GENERAL.COMPRESS TRUE [current_design] set_property BITSTREAM.CONFIG.CONFIGRATE 33 [current_design] set_property CONFIG_MODE SPIx4 [current_design]

RTL Schematic:

Vivado RTL Synthesis & Implementation Summary: The design was synthesized and implemented using Vivado 2020.1 targeting the Artix-7 xc7a35tcpg236-1 FPGA. The schematic confirms hierarchical integration of the debouncer, clock divider, addressDecoderTop (DataMemory, AddressDecoder, leds, switches), and 7-segment display controller. Timing analysis passed with positive slack (WNS = 1.860 ns) and bitstream top_memory_system.bit was generated with 0 errors.

Vivado RTL Synthesis & Implementation Summary: The design was synthesized and implemented using Vivado 2020.1 targeting the Artix-7 xc7a35tcpg236-1 FPGA. The schematic confirms hierarchical integration of the debouncer, clock divider, addressDecoderTop (DataMemory, AddressDecoder, leds, switches), and 7-segment display controller. Timing analysis passed with positive slack (WNS = 1.860 ns) and bitstream top_memory_system.bit was generated with 0 errors.

# Lab 08 – Design and FPGA Implementation of Memory System

# with Address Decoding for a Single-Cycle RISC-V Processor

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Task 1 | - | - | /20 | /20 |
| Task 2 | /20 | /15 |  |  |
| Task 3 | /10 | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |